# Module 3 — The Data Link Layer (Framing, Error & Flow Control)

Computer Networks Mastery: SEBI IT / RBI / NABARD / GATE / C-DAC / Interview / Backend — In-Depth Notes (with visuals)

Prepared for nidhi.sharma@forensiccybertech.com

2026

Table of Contents

# Module 3 — The Data Link Layer (Framing, Error & Flow Control)

Where this module sits. The Physical layer (M2) moves raw bits but makes no promises — bits can flip, and a fast sender can drown a slow receiver. Layer 2, the Data Link layer (DLL), turns that unreliable bit-pipe into a reliable link between two directly-connected nodes. It groups bits into frames, detects and corrects errors, and controls the flow so nobody is overwhelmed. This is one of the most numerical-heavy modules in the syllabus — CRC, Hamming code, and sliding-window efficiency calculations appear in almost every GATE/C-DAC paper.

Importance ratings (out of 5):

| Exam / Use | SEBI IT | RBI IT | GATE CS | Interview | Backend |
| --- | --- | --- | --- | --- | --- |
| This module | ★★★ | ★★★ | ★★★★★ | ★★ | ★★ |

Most-asked PYQ concepts (SEBI / RBI / GATE / C-DAC): CRC computation (polynomial division); Hamming code (number of parity bits, error position); framing (bit stuffing / byte stuffing); stop-and-wait vs sliding window efficiency (U = 1/(1+2a)); Go-Back-N vs Selective Repeat (window sizes 2^m−1 vs 2^(m−1)); piggybacking; parity vs checksum vs CRC (detection power).

## 3.1 What the Data Link Layer Does (first principles)

The DLL provides node-to-node delivery over one link. Its jobs:

- Framing — group bits into frames with clear start/end.

- Physical addressing — put source/destination MAC addresses on the frame (details in M4).

- Error control — detect (and sometimes correct) bit errors.

- Flow control — stop a fast sender from overwhelming a slow receiver.

- Access control — decide who may transmit when the medium is shared (M4).

The DLL has two sublayers: LLC (Logical Link Control — flow/error control) and MAC (Media Access Control — addressing + who-talks-when, covered in M4).

Memory hook: the DLL is a careful courier for one leg of the journey — it boxes the goods (framing), labels them (MAC), checks nothing broke (error control), and doesn’t hand over faster than the receiver can take (flow control).

### MCQs

- The DLL delivers between? → two directly-connected nodes (node-to-node).

- Two DLL sublayers? → LLC and MAC.

- Which layer adds the MAC address? → Data Link (2).

## 3.2 Framing — Where Does a Frame Start and End?

The receiver sees a stream of bits; framing marks boundaries. Methods:

- Character/byte count: first field says how many bytes follow. Fragile — if the count is corrupted, all following frames desync.

- Byte (character) stuffing: mark frame with a FLAG byte; if the flag appears in data, insert an escape (ESC) byte before it (and escape the ESC too).

- Bit stuffing: frame delimited by the flag 01111110. In the data, after five consecutive 1s the sender inserts a 0; the receiver removes it. This guarantees the flag never appears inside data.

Bit-stuffing worked example: data 0111111 0111110 → after every five 1s insert a 0 → 011111**0**1 0111110 (sender stuffs a 0 after the first five 1s). The receiver strips the stuffed 0 to recover the original.

Memory hook: bit stuffing = “after five 1s, sneak in a 0” so the real flag 01111110 is never faked by the data.

### MCQs

- Flag pattern in bit stuffing? → 01111110.

- Bit-stuffing rule? → after five consecutive 1s, insert a 0.

- Byte stuffing uses which special byte before a data-flag? → ESC.

## 3.3 Error Detection — Parity, Checksum, CRC

Noise flips bits. Detection adds redundant bits so the receiver can tell something changed (it can’t necessarily fix it).

### Parity

- Simple (1-D) parity: add one bit so the total number of 1s is even (even parity). Detects any odd number of bit errors; misses even-numbered errors.

- 2-D parity: parity per row and per column — detects and can locate many errors.

### Checksum

Sum the data words (with end-around carry), send the complement; receiver re-adds everything and expects all-1s. Used by IP/TCP/UDP headers. Weaker than CRC.

### CRC (Cyclic Redundancy Check) — the workhorse

Treats the bit string as a polynomial and divides by a generator polynomial (using XOR / modulo-2 arithmetic). The remainder (CRC) is appended. The receiver divides the whole thing by the same generator — a non-zero remainder means error.

CRC worked example
  Data      : 1101011011           Generator G : 10011  (degree 4 -> append 4 zeros)
  Dividend  : 1101011011 0000      (data << 4)

  Modulo-2 (XOR) long division by 10011:
    1101011011 0000  ÷ 10011
    ... performing XOR division ...
    Remainder (CRC) = 1110

  Transmit  : 1101011011 1110      (data + CRC)
  Receiver  : divides received frame by 10011 ->
              remainder 0000 = OK ;  non-zero = ERROR detected.

- A CRC with an r-bit remainder needs a generator of degree r (r+1 bits) and appends r zero bits to the data before dividing.

- CRC catches all single-bit errors, all odd errors (if G has a factor x+1), all burst errors shorter than r, and most longer ones — far stronger than checksum.

Memory hook: CRC = binary long division with XOR; remainder 0 at the receiver = clean. Degree of generator = number of CRC bits = zeros you append.

### MCQs

- Simple parity misses which errors? → an even number of bit flips.

- CRC uses which arithmetic? → modulo-2 (XOR) polynomial division.

- Generator of degree r → how many CRC bits / appended zeros? → r.

- Receiver’s remainder is 0 → ? → no error detected.

## 3.4 Error Correction — Hamming Code

Detection just says “something broke.” Hamming code can correct a single-bit error by adding parity bits at power-of-two positions.

Hamming code places parity bits at positions 1,2,4,8…; each parity checks a fixed set of positions, so the failed checks spell out the error’s position in binary.

- Number of parity bits r for m data bits: choose the smallest r with 2^r ≥ m + r + 1.

- Placement: parity bits go at positions 1, 2, 4, 8, … (powers of 2); data fills the rest.

- Each parity covers positions whose binary index includes its bit: P1 checks 1,3,5,7…; P2 checks 2,3,6,7…; P4 checks 4,5,6,7,12….

- At the receiver: recompute each parity; the binary number formed by the failing checks is the position of the wrong bit — flip it to correct.

Worked example: for m = 4 data bits, 2^r ≥ 4 + r + 1 → r = 3 (since 2³ = 8 ≥ 8). So Hamming(7,4): 7 total bits, 4 data + 3 parity, corrects any single-bit error.

Memory hook: parity bits sit at powers of two; the syndrome (which checks fail, read as binary) points straight at the broken bit.

### Detection vs correction (a classic distinction)

- To detect up to d errors, you need Hamming distance d + 1.

- To correct up to d errors, you need Hamming distance 2d + 1.

### MCQs

- Parity bits for 4 data bits? → 3 (Hamming(7,4)).

- Parity-bit positions? → powers of 2 (1,2,4,8…).

- Hamming distance to correct d errors? → 2d + 1.

- The failing-checks binary number gives? → the error position.

## 3.5 Flow Control & ARQ — Stop-and-Wait

Flow control matches sender speed to receiver capacity. ARQ (Automatic Repeat reQuest) adds reliability: lost/corrupt frames are retransmitted using ACKs and timeouts.

Stop-and-Wait: the sender transmits one frame and then waits for its ACK before sending the next, leaving the link idle for a full round trip.

Stop-and-Wait ARQ: send one frame, wait for its ACK, then send the next. Simple, but the link is idle during the round trip → poor efficiency on long/fast links.

Efficiency (the key formula). Let a = Tp / Tt where Tt = transmission time (frame/bandwidth) and Tp = propagation time:

Stop-and-Wait efficiency  U = Tt / (Tt + 2·Tp) = 1 / (1 + 2a)
Throughput = U × Bandwidth

Worked example: bandwidth 1 Mbps, frame 1000 bits, Tp = 10 ms → Tt = 1000/10⁶ = 1 ms, a = 10/1 = 10 → U = 1/(1 + 20) = 1/21 ≈ 4.76%. The long link is almost idle — motivating sliding windows.

### MCQs

- Stop-and-Wait efficiency? → 1/(1 + 2a), a = Tp/Tt.

- Why is Stop-and-Wait poor on long links? → link idle for the whole round trip.

- a is defined as? → Tp / Tt (propagation / transmission).

## 3.6 Sliding Window — Go-Back-N & Selective Repeat

To keep the pipe full, allow N unacknowledged frames in flight (a window).

Go-Back-N discards out-of-order frames and resends everything from the lost frame; Selective Repeat buffers out-of-order frames and resends only the lost one.

### Go-Back-N (GBN)

- Sender may send up to N frames without ACK; receiver accepts only in order and discards anything out of order.

- On a loss/timeout, the sender goes back and resends the lost frame and all after it. Simple receiver (no buffering), but wasteful on lossy links.

- Sender window ≤ 2^m − 1 (m = sequence-number bits). Receiver window = 1.

### Selective Repeat (SR)

- Receiver buffers out-of-order frames and ACKs them individually; sender resends only the specific lost frame. Efficient but needs buffering + more bookkeeping.

- Sender & receiver window ≤ 2^(m−1) (must be ≤ half the sequence space to avoid ambiguity).

### Efficiency (sliding window)

U = N / (1 + 2a)      capped at 1  (when N ≥ 1 + 2a, the pipe is full, U = 100%)
  N = window size,  a = Tp/Tt
Optimal window to fill the pipe:  N = 1 + 2a

Worked example: with a = 10 (from §3.5), Stop-and-Wait (N=1) gave 4.76%. To fill the pipe we need N = 1 + 2×10 = 21 frames in flight → U = 100%. This is why sliding windows exist.

Memory hook: GBN = “go back and redo from the mistake” (simple, wasteful). SR = “just redo the one that broke” (efficient, needs buffers). Window sizes: GBN 2^m−1, SR 2^(m−1).

### MCQs

- GBN max sender window? → 2^m − 1. SR window? → 2^(m−1).

- Sliding-window efficiency? → N/(1 + 2a) (max 1).

- Which protocol buffers out-of-order frames? → Selective Repeat.

- Window size N to fully utilise the link? → N = 1 + 2a.

## 3.7 Piggybacking & Other Essentials

- Piggybacking: on a two-way link, attach the ACK for received data onto an outgoing data frame instead of sending a separate ACK — saves bandwidth (small delay while waiting for a data frame to ride on).

- NAK (negative ACK): explicitly requests a resend (used by Selective Repeat).

- Cumulative ACK: one ACK confirms all frames up to a number (GBN, TCP).

### MCQs

- Combining an ACK with an outgoing data frame? → piggybacking.

- “ACK N” confirming everything up to N is a? → cumulative ACK.

## 3.8 Real-World & Backend Perspectives

- Ethernet frames (M4) are the DLL in practice: preamble, MAC addresses, type/length, payload, and a CRC-32 trailer (FCS) — exactly the framing + error detection here.

- The sliding window you learn here is the same idea TCP uses at Layer 4 (M8) — a window of unacknowledged data sized to the bandwidth-delay product (M2). DLL = one link; TCP = end-to-end, but the mechanism rhymes.

- Wi-Fi uses link-layer ARQ (retransmit lost frames) because radio is lossy — which is why Wi-Fi latency is jittery.

## 3.9 Tradeoffs, Common Mistakes, Edge Cases

- Mistake: confusing detection (parity/checksum/CRC — “something’s wrong”) with correction (Hamming — “fix it”). CRC does not correct.

- Mistake: SR window = 2^m − 1. It’s 2^(m−1) (half) — over-sizing causes sequence-number ambiguity.

- Trap: forgetting to append r zeros to the data before CRC division.

- Edge case: parity misses even-count errors; that’s why real links use CRC.

## 3.10 Exam, Interview & Coding Perspectives

- SEBI / RBI / NABARD: conceptual — framing methods, parity vs CRC vs Hamming, what piggybacking is, GBN vs SR difference.

- GATE / C-DAC: the numericals dominate — CRC remainder, Hamming parity-bit count & error position, stop-and-wait / sliding-window efficiency, window-size formulas. Drill these.

- Interview: “how does a sliding window improve throughput?” → the efficiency formula and bandwidth-delay-product intuition.

## 3.11 Concept Checks & MCQs (test yourself)

- Bit-stuffing rule and flag? → insert 0 after five 1s; flag 01111110.

- Generator degree 4 → CRC bits and appended zeros? → 4.

- Parity bits for 8 data bits? → smallest r with 2^r ≥ 8 + r + 1 → r = 4.

- Stop-and-Wait efficiency with a = 3? → 1/(1 + 6) = 1/7 ≈ 14.3%.

- GBN vs SR sender window (m bits)? → 2^m − 1 vs 2^(m−1).

- Sliding-window efficiency, N = 5, a = 2? → 5/(1 + 4) = 1 (100%).

- Which corrects a single-bit error? → Hamming code.

- Which is stronger detection: parity or CRC? → CRC.

- Combining ACK with a data frame? → piggybacking.

- Receiver discards out-of-order frames in? → Go-Back-N.

## 3.12 One-Page Revision Sheet

DLL (L2): node-to-node over ONE link. PDU=FRAME. Sublayers: LLC(flow/error)+MAC(address/access).
JOBS: framing, MAC addressing, error control, flow control, access control.

FRAMING: char-count(fragile) | byte stuffing(FLAG + ESC) | bit stuffing(after five 1s insert 0; flag 01111110).

ERROR DETECTION:
  parity: even-parity; catches ODD errors, misses EVEN. 2-D locates errors.
  checksum: sum + complement; used by IP/TCP/UDP; weak.
  CRC: modulo-2 (XOR) poly division by generator G(degree r). append r zeros.
       remainder=CRC; receiver remainder 0 => OK. catches bursts < r. STRONGEST.

ERROR CORRECTION (Hamming): parity at positions 1,2,4,8...(powers of 2).
  parity bits r: smallest r with 2^r >= m+r+1.  m=4 -> r=3 => Hamming(7,4).
  syndrome (failed checks in binary) = ERROR POSITION -> flip it.
  detect d errors: distance d+1 ; correct d: distance 2d+1.

FLOW/ARQ:  a = Tp/Tt.
  Stop-and-Wait U = 1/(1+2a).
  Sliding window U = N/(1+2a) capped 1. Full pipe when N = 1+2a.
  GBN: receiver in-order only, DISCARDS oob, resend-all-after-loss. Wsend <= 2^m - 1.
  SR : receiver BUFFERS oob, resend-only-lost. Wsend=Wrecv <= 2^(m-1).
  PIGGYBACKING = ACK rides on outgoing data. Cumulative ACK (GBN/TCP). NAK (SR).

### Flash cards

| Front | Back |
| --- | --- |
| Bit-stuffing rule | after five 1s, insert a 0 (flag 01111110) |
| CRC arithmetic | modulo-2 XOR polynomial division |
| CRC bits for gen degree r | r (append r zeros) |
| Hamming parity count | smallest r: 2^r ≥ m+r+1 |
| Hamming parity positions | powers of 2 (1,2,4,8) |
| Correct d errors needs distance | 2d + 1 |
| Stop-and-Wait efficiency | 1/(1+2a), a = Tp/Tt |
| Sliding-window efficiency | N/(1+2a), max 1 |
| GBN vs SR window | 2^m−1 vs 2^(m−1) |
| Piggybacking | ACK carried on a data frame |

### Spaced repetition

- 24-hour: compute one CRC remainder and one Hamming parity-bit count from scratch.

- 7-day: stop-and-wait & sliding-window efficiency numericals; GBN vs SR windows.

- 30-day: given bandwidth/frame/Tp, find a, the efficiency, and the window size to reach 100% — without notes.

## 3.13 Summary

The Data Link layer turns the raw bit-pipe into a reliable link between two nodes. It performs framing (bit/byte stuffing), error detection (parity < checksum < CRC, the strongest, via modulo-2 division), and error correction (Hamming code, parity bits at powers of two, syndrome = error position). It controls flow and reliability with ARQ: Stop-and-Wait (efficiency 1/(1+2a), poor on long links) and sliding-window protocols — Go-Back-N (resend everything after a loss, window 2^m−1) and Selective Repeat (resend only the lost frame, window 2^(m−1)), with efficiency N/(1+2a). Extras like piggybacking and cumulative ACKs save bandwidth. These mechanisms reappear end-to-end in TCP (M8).

Next, Module 4 — The MAC Sublayer & LANs answers “who gets to talk on a shared medium?” — ALOHA, CSMA/CD (Ethernet), CSMA/CA (Wi-Fi), plus switching, ARP, and VLANs.

You have mastered this module when you can: stuff/unstuff bits, compute a CRC remainder and verify it, size and place Hamming parity bits and locate an error from the syndrome, derive stop-and-wait and sliding-window efficiency from a, and state the GBN vs SR window formulas and behaviours — all without notes.